Simulink-Engduino Tutorial

- Introduction to Simulink

(with version Matlab 2015a)

Simulink is a visual dataflow language. Programs are implemented by connecting various blocks together. Blocks are of 3 main types:

- Sources(e.g. constants, inputs, sensors) - these blocks represent entry points of data into the program and only have output ports

- Sinks(e.g. displays, outputs, LEDs) - these blocks represent output points of the program’s data and only have input ports

- Transforms(e.g. mathematical functions) - these blocks represent different operations on data and have both input and output ports

Blocks can be connected by clicking on the output arrow of a block, which will bring up a wiring tool. The Engduino library consists of a collection of sources and sinks that can be connected together by various built-in Simulink transforms and then deployed on the board.

Sink block

Transform blocks

Source block

- Getting Start - Installation

Make sure you have a valid Simulink license. Start MATLAB.

# a) Install Hardware support package

Before we can use Engduino with Simulink, we need to install a Arduino hardware support package.

From the top menu bar, choose Add-On>>Get Hardware Support Packages.

Choose Install from Internet, then next

You will see an option to Arduino, choose all three packages and press next.

Accept the license terms and conditions and install the package.

b) Download Engduino Matlab examples

You can download the entire Engduino support to date from this location:-

https://github.com/engduino/Engduino-Extensions

This contains a MatlabSupport folder with examples running Matlab with Engduino; and a SimulinkSupport folder with Simulink Engduino examples and codes. Unzip the folder to a location on your computer. (e.g. My Documents\MATLAB)

- Open an example

From the MATLAB top toolbar menu, choose Open, navigate to the SimulinkSupport folder you have downloaded in Step 2b). Choose examples1.slx to open. You will see these Simulink blocks.

Before we can program the Engduino with Simulink, we need to do some configurations. Navigate to Tools>>Run on Hardware Target>>Options

Choose Arduino LilyPad USB as Target hardware. The default host COM port should be Automatically. Leave this and other setting unchanged. Apply this setting and click OK.

- Download the blocks to Engduino

Simply press the Deploy to hardware button to download the blocks onto Engduino.

- Engduino blocks libraries

In R2015a, Engduino is not yet an integrated part of Simulink. Hence you would find the available Engduino blocks in engduino_drivers.slx. Simply copy the blocks you want to use to your own Simulink model.

In the future, you would be able to use the Library Browser in Simulink to add the blocks.